3D Woodpile Photonic Crystal

Quantum Entanglement Protocol

NV Diamond Implementation at Room Temperature

Applied from photonic circuit QED base protocol

# Overview

This document applies the four-step photonic crystal quantum entanglement circuit to NV diamond centres operating at room temperature inside a 3D microwave woodpile photonic crystal cavity. Each step maps the original photon-based operation onto the physical reality of NV spin dynamics, optical initialisation, and microwave spin control.

The key distinction from the base protocol is the qubit carrier. In the original circuit, the qubit is a microwave photon number state. In the NV implementation, the qubit is the NV electron spin state. The photonic crystal cavity becomes the bus that routes quantum information between spin qubits rather than storing it.

| Parameter | Original protocol | NV diamond room temp |
| --- | --- | --- |
| Qubit carrier | Microwave photon | NV electron spin (ms = 0, ±1) |
| Initialisation | π pulse via microwave | 532 nm optical pumping |
| Qubit frequency | Cavity resonance ~5 GHz | 2.87 GHz (zero field) |
| Readout | Transmission line homodyne | 637 nm fluorescence counting |
| Coherence time | Cavity photon lifetime | ~1–100 µs (spin T2) |
| Temperature | ~20 mK required | Room temperature viable |
| Coupling regime | Strong (g > κ) | Weak (ensemble needed) |

# Step-by-Step Protocol

Step 1: Prepare spin state in cavity A

Original (photon circuit QED): π pulse via microwave control line prepares ¦1⟩ photon state in cavity A. NV diamond at room temperature: Shine 532 nm green laser on diamond A for 1–10 µs. NV centre optically pumped into ¦ms=0⟩ ground state (~95% fidelity). Then apply microwave π/2 pulse at 2.87 GHz to create superposition: ¦0⟩ → (¦0⟩ + ¦1⟩)/√2.

Key details for Step 1:

- Laser wavelength: 532 nm (green). Any DPSS or diode laser at this wavelength works.

- Microwave frequency: 2.87 GHz at zero external magnetic field. Shift with B-field to tune.

- Pulse duration: π/2 pulse length determined by Rabi frequency — measure via ODMR first.

- Fidelity: ~95% initialisation at room temp. Improves to ~99%+ at cryogenic temperatures.

- Repeat for cavity B: initialise NV_B into |0⟩, leave there until Step 4.

Step 2: Transduce spin state into cavity photon

Original (photon circuit QED): Open waveguide coupling for time t = π/(2g). Exactly half a photon exchange creates 50/50 superposition across cavities A and B. NV diamond at room temperature: Tune NV_A transition into resonance with cavity A mode using external magnetic field. Spin–photon SWAP maps spin superposition onto cavity field. Photon then propagates through line defect waveguide to cavity B. Use NV ensemble (N~10⁶ centres) to achieve g√N > κ in weak coupling regime.

This is the hardest step at room temperature. The challenge is achieving sufficient coupling strength before the cavity photon decays. Three approaches:

| Approach | How it works | Room temp viable? |
| --- | --- | --- |
| NV ensemble | Collective coupling √N × g. 10⁶ spins gives 1000× boost. | Yes — primary method |
| Pulsed SWAP | Fast π pulse faster than cavity decay. Requires g ~ κ. | Marginal — needs high Q |
| Heralded transfer | Post-select on successful photon arrival. Probabilistic but clean. | Yes — slower but reliable |

Waveguide propagation: photon travels through line defect (row of modified rods) guided by bandgap. Copper woodpile at room temp will have high loss here. Test with VNA first — measure S21 between two defect ports to confirm routing before adding diamonds.

Step 3: Apply phase gate to NV_B spin

Original (photon circuit QED): Apply phase shift to cavity B via pump tone — controlled-Z phase gate. NV diamond at room temperature: After photon absorbed by NV_B ensemble (reverse of Step 2), apply microwave phase pulse directly to NV_B spin. Phase φ = π for Bell state. This is a single-qubit phase gate on the spin: ¦1⟩_B → e^(iφ)¦1⟩_B.

This step is the simplest to implement. A microwave pulse with controlled phase is standard RF electronics. The parameters:

- Frequency: 2.87 GHz (or tuned value with B-field)

- Phase: φ = π for maximally entangled Bell state

- Duration: π pulse duration (same as calibrated from ODMR)

- Hardware: any vector signal generator with IQ modulation

The combined two-spin state after this gate, assuming successful photon transfer, is:

(|00⟩ + |01⟩ + |10⟩ + |11⟩)/2

↓ phase gate on B (φ = π)

(|00⟩ + |01⟩ − |10⟩ − |11⟩)/2  →  Bell state |Φ⁻⟩

Step 4: Optical readout and Bell state verification

Original (photon circuit QED): Read out via shared transmission line — heralded Bell state confirmed. NV diamond at room temperature: Shine 637 nm red laser on both diamonds simultaneously. Bright fluorescence = ¦ms=0⟩. Dim fluorescence (~70% of bright) = ¦ms=±1⟩. Correlate photon counts from cavity A and cavity B detectors. Bell state violation (S > 2 in CHSH inequality) confirms entanglement.

Readout hardware requirements:

- Laser: 637 nm (red). Can use same 532 nm laser with appropriate filter, or dedicated red laser.

- Detector: single photon avalanche diode (SPAD) or photomultiplier tube per cavity.

- Contrast: ~30% at room temp between bright and dim states.

- Averaging: ~100 shots per measurement point for reliable discrimination at room temp.

- Single-shot options: spin-to-charge conversion (room temp), or repetitive nuclear spin readout (~91% fidelity).

Bell inequality test (CHSH):

Measure spin correlations in four basis combinations: ZZ, ZX, XZ, XX. Compute S = |E(ZZ) + E(ZX) + E(XZ) − E(XX)|. Classical limit: S ≤ 2. Quantum violation: S up to 2√2 ≈ 2.83. Any S > 2 confirms entanglement.

# Full Circuit Summary

| Step | Operation | Hardware | Duration |
| --- | --- | --- | --- |
| 1a | Initialise NV_A and NV_B | 532 nm laser, ~10 mW | 1–10 µs |
| 1b | Superpose NV_A | MW source at 2.87 GHz, π/2 pulse | 10–100 ns |
| 2a | Tune NV_A to cavity resonance | External B-field coil | Static |
| 2b | Spin → photon SWAP (NV_A) | MW pulse + cavity coupling | ~1/g µs |
| 2c | Photon routes to cavity B | Line defect waveguide | ~L/c ns |
| 2d | Photon → spin SWAP (NV_B) | Cavity coupling + MW pulse | ~1/g µs |
| 3 | Phase gate on NV_B spin | MW phase pulse, φ = π | 10–100 ns |
| 4a | Readout NV_A | 637 nm laser + SPAD | ~300 ns per shot |
| 4b | Readout NV_B | 637 nm laser + SPAD | ~300 ns per shot |
| 4c | Correlate and verify Bell state | FPGA / time tagger | Post-processing |

# What to Test Right Now (Copper, Room Temperature)

You do not need a dilution fridge to begin. The following sequence validates each part of the protocol before committing to expensive hardware:

| Test | Equipment | What it confirms |
| --- | --- | --- |
| ODMR on diamond | 532 nm laser, MW antenna, photodetector | NV working, find exact 2.87 GHz transition |
| Rabi oscillations | Pulsed MW source, time-resolved fluorescence | Coherent spin control, calibrate π pulse |
| Cavity transmission (S21) | Vector network analyser, 2 ports | Defect mode frequency, Q-factor, bandgap |
| NV-cavity avoided crossing | VNA + B-field magnet | Coupling rate g between spin and cavity |
| Waveguide S21 (A to B) | VNA across both defect ports | Photon routing loss between cavities |
| Parametric pump response | MW pump at f_A + f_B | Classical precursor to entanglement correlations |

Start with ODMR. It requires only a 532 nm laser, a small microwave loop antenna near the diamond, and a photodetector. Total hardware cost ~£500–£2000 depending on what you already have. If ODMR works, the NV centres are functional and the rest follows.

# Limitations at Room Temperature

| Limitation | Cause | Mitigation |
| --- | --- | --- |
| Cavity flooded with thermal photons | kT >> hf at 300 K, ~1200 photons at 5 GHz | Heralded protocols, post-selection |
| Low cavity Q (copper) | Normal metal surface resistance ~10 mΩ/sq | Ensemble coupling, or Al insert at defect |
| NV spin T2 ~1–100 µs | Phonon dephasing, charge noise at 300 K | Dynamical decoupling (CPMG sequences) |
| Weak single-spin coupling | g ~ kHz for single NV to microwave cavity | Ensemble of 10⁶ NV centres, g_eff ~ MHz |
| 30% readout contrast | Spin-state mixing at room temp | Repetitive readout, or spin-to-charge conversion |
| No quantum Bell violation likely | Thermal decoherence faster than gate | Demonstrate classical correlations first, then cool |

# Upgrade Path

- Current stage: Copper woodpile, room temperature — characterise bandgap and defect modes with VNA

- Stage 2: Add NV diamond to defect, room temperature — ODMR, Rabi, NV-cavity coupling

- Stage 3: Replace copper rods with aluminium — Q improves 3–4 orders of magnitude

- Stage 4: Cool to 4K (liquid helium) — thermal photon number drops to ~17, coherence improves

- Stage 5: Cool to 20 mK (dilution fridge) — thermal photon number ~0.002, quantum regime

- Stage 6: Demonstrate spin–photon entanglement, Bell inequality violation

- Stage 7: Scale to multiple defect cavity pairs, implement bosonic error correction

Each stage produces publishable results. Stage 2 alone (NV-microwave woodpile coupling) would be a novel experimental result with no direct prior art.